# Display Hardware Profiles

Minimal practical guide for BACnet-ESP32-S3-SEN54-ST7796S-DS18B2

Branch: multi

Purpose: Keep one project while selecting a complete display configuration at compile time: UI source, controller, GPIO mapping, dimensions, backlight, and SPI settings.

# 1. How it works

One menuconfig choice drives two build paths:

CONFIG_USER_DISPLAY_ST7796S_TEST=y ¦ +--> main/CMakeLists.txt ¦ compiles display_st7796s_test.cpp ¦ +--> components/TFT_eSPI/User_Setup.h includes Setup_Project_ST7796S.h

`display.h` is the stable API used by the application.

`display_*.cpp` contains UI layout and drawing logic.

`Setup_Project_*.h` contains controller, GPIO, SPI and panel settings.

Only one UI implementation is compiled in each build.

# 2. Required file structure

main/ ├── Kconfig.projbuild ├── CMakeLists.txt └── ui/ ├── display.h └── profiles/ ├── display_st7796s_current.cpp ├── display_st7796s_test.cpp └── display_none.c components/TFT_eSPI/ ├── User_Setup.h └── User_Setups/ └── Setup_Project_ST7796S.h

# 3. Profile selection files

## Kconfig: available profiles

menu "Project hardware" choice USER_DISPLAY_PROFILE prompt "Display profile" default USER_DISPLAY_ST7796S_CURRENT config USER_DISPLAY_ST7796S_CURRENT bool "ST7796S 480x320 - current UI" config USER_DISPLAY_ST7796S_TEST bool "ST7796S 480x320 - test UI" endchoice endmenu

## CMake: compile one UI source

if(CONFIG_USER_DISPLAY_ST7796S_CURRENT) list(APPEND APP_SRCS "ui/profiles/display_st7796s_current.cpp" ) elseif(CONFIG_USER_DISPLAY_ST7796S_TEST) list(APPEND APP_SRCS "ui/profiles/display_st7796s_test.cpp" ) elseif(NOT CMAKE_BUILD_EARLY_EXPANSION) message(FATAL_ERROR "No display profile selected") endif()

Important: Keep the CMAKE_BUILD_EARLY_EXPANSION guard. ESP-IDF reads component CMake files before Kconfig values are available during its preliminary dependency scan.

## TFT_eSPI selector: choose the hardware setup

#pragma once #include "sdkconfig.h" #if defined(CONFIG_USER_DISPLAY_ST7796S_CURRENT) ¦¦ \ defined(CONFIG_USER_DISPLAY_ST7796S_TEST) #include "User_Setups/Setup_Project_ST7796S.h" #else #error "No TFT_eSPI hardware setup selected" #endif

# 4. Hardware setup file

The hardware setup describes the physical panel and wiring. The current display uses:

#pragma once #define USER_SETUP_INFO "Project ST7796S 320x480 SPI" #define ST7796_DRIVER #define TFT_WIDTH 320 #define TFT_HEIGHT 480 #define TFT_MOSI 10 #define TFT_SCLK 9 #define TFT_MISO -1 #define TFT_CS 13 #define TFT_DC 12 #define TFT_RST 11 #define TFT_BL 14 #define TFT_BACKLIGHT_ON HIGH #define USE_HSPI_PORT #define SPI_FREQUENCY 26000000 #define TFT_RGB_ORDER TFT_BGR #define TFT_INVERSION_OFF #define LOAD_GLCD #define LOAD_FONT2 #define LOAD_FONT4 #define LOAD_GFXFF #define SMOOTH_FONT #define TOUCH_CS -1

Dimensions: TFT_WIDTH and TFT_HEIGHT are the native panel dimensions. Landscape orientation belongs in the UI file through tft.setRotation(...).

# 5. Rules for every UI profile

Implement exactly the functions declared in `main/ui/display.h`.

Keep UI coordinates, text, colours and rotation in the profile source.

Do not hardcode controller pins or backlight GPIOs in the UI source.

Use the selected setup macros such as `TFT_BL` and `TFT_BACKLIGHT_ON`.

Add a unique startup log so the compiled profile is obvious.

## Required logging declaration

#include "esp_log.h" static const char *TAG = "display";

## Backlight handling

#if defined(TFT_BL) && (TFT_BL >= 0) pinMode(TFT_BL, OUTPUT); digitalWrite(TFT_BL, TFT_BACKLIGHT_ON); delay(10); #endif

## Startup diagnostics

tft.init(); tft.setRotation(1); ESP_LOGI(TAG, "UI profile: ST7796S TEST"); ESP_LOGI(TAG, "TFT hardware: %s", USER_SETUP_INFO); ESP_LOGI(TAG, "GPIO MOSI=%d SCLK=%d CS=%d DC=%d RST=%d BL=%d", TFT_MOSI, TFT_SCLK, TFT_CS, TFT_DC, TFT_RST, TFT_BL);

# 6. Add a new hardware profile

1. Create a new feature branch.

2. Confirm the exact display controller from a datasheet or known working example.

3. List all GPIOs already used by sensors, USB, flash/PSRAM, UART and future RS485.

4. Duplicate the closest existing `display_*.cpp` file and give it a unique profile log.

5. Add one new entry inside the Kconfig display `choice`.

6. Add one corresponding `elseif()` branch in `main/CMakeLists.txt`.

7. Create a new `Setup_Project_*.h` when the controller or wiring differs.

8. Add the new profile to `TFT_eSPI/User_Setup.h`.

9. Start an untested display at a conservative SPI frequency, such as 10 MHz.

10. Run Full Clean, build, flash and verify the profile and GPIOs in the serial log.

## Example: new ST7789 profile

Kconfig symbol: USER_DISPLAY_ST7789_240X320 UI source: main/ui/profiles/display_st7789_240x320.cpp Hardware setup: components/TFT_eSPI/User_Setups/ Setup_Project_ST7789_240x320.h Controller macro: #define ST7789_DRIVER Native dimensions: #define TFT_WIDTH 240 #define TFT_HEIGHT 320

One controller only: A TFT_eSPI hardware setup must enable exactly one controller macro. Screen size alone does not identify the controller.

# 7. Select, build and test

1. In VS Code press Ctrl+Shift+P.

2. Run `ESP-IDF: SDK Configuration Editor (menuconfig)`.

3. Open `Project hardware` > `Display profile`.

4. Select the desired profile and save.

5. Run `ESP-IDF: Full Clean Project` after hardware, Kconfig or TFT_eSPI setup changes.

6. Build, flash and monitor.

7. Confirm the expected UI profile, hardware name and GPIOs in the startup log.

A normal build is enough for layout-only changes. Use Full Clean after changing controller, GPIOs, SPI settings, native dimensions, enabled fonts, Kconfig, or CMake source selection.

# 8. Minimal troubleshooting

| Problem | Check / fix |
| --- | --- |
| 'TAG' was not declared | Include esp_log.h and add static const char *TAG = "display"; |
| No profile or TFT setup selected | Check the same Kconfig symbol in Kconfig.projbuild, CMakeLists.txt and User_Setup.h; then run Full Clean. |
| Multiple definition of display_init | CMake is compiling more than one display profile. |
| Undefined reference to display_init | No UI profile was compiled or its function signature does not match display.h. |
| Blank screen or wrong colours | Check controller, power, BL polarity, CS/DC/RST, MOSI/SCLK, low SPI speed, RGB order and inversion. |
| Old profile/setup still used | Run ESP-IDF: Full Clean Project, then rebuild and verify sdkconfig and startup logs. |

# 9. New-profile checklist

Unique Kconfig choice added.

Exactly one UI source selected by CMake.

Matching hardware setup selected in User_Setup.h.

Correct controller, native size and GPIO assignments.

No GPIO conflicts.

Backlight uses TFT_BL and TFT_BACKLIGHT_ON.

Unique profile startup log present.

Full clean completed.

Basic fill-screen test works before adapting the complete UI.

Sensor and BACnet behavior remains unchanged.

Future RS485 profiles: Use the same pattern later: one Kconfig choice, one selected implementation, and one GPIO/transceiver configuration. Keep display and RS485 choices separate until stable board combinations are known.